Material Safety Data Sheet

- MATERIAL AND COMPANY IDENTIFICATION

Material Name 		: Petro Grease MP 3

Uses				: Automotive and industrial grease.

Manufacturer/Supplier	 :  PT. PETROMITRA PACIFIC INTERNUSA

Jl Raya Serpong No 26B

Tangerang

Emergency Telephone Number

Spill Information		 : 021-53125041

Health Information 		 : 021-53125041

- COMPOSITION/INFORMATION ON INGREDIENTS

Chemical Identity 			CAS No. 			Concentration

Asphalt, fumes 			8052-42-4 			1.00 - 5.00%

A lubricating grease consisting of highly-refined mineral oil and additives.

The highly refined mineral oil contains <3% (w/w) DMSO-extract, according to IP346.

- HAZARDS IDENTIFICATION

Emergency Overview

Appearance and Odour 	: Transparant Brown. Semi-solid. Slight hydrocarbon.

Health Hazards		: High-pressure injection under the skin may cause serious

damage including local necrosis.

Safety Hazards 		: Not classified as flammable but will burn.

Environmental Hazards 	: Not classified as dangerous for the environment.

Health Hazards 		: Not expected to be a health hazard when used under

Normal conditions.

Health Hazards

Inhalation			 : Under normal conditions of use, this is not expected to be a

primary route of exposure.

Skin Contact 			:  Prolonged or repeated skin contact without proper cleaning can

clog the pores of the skin resulting in disorders such as oil

acne/folliculitis.

Eye Contact 			:  May cause slight irritation to eyes.

Ingestion			:  Low toxicity if swallowed.

Other Information 		: High-pressure injection under the skin may cause serious

damage including local necrosis. Used grease may contain

harmful impurities.

Signs and Symptoms	: Local necrosis is evidenced by delayed onset of pain and tissue

damage a few hours following injection. Oil acne/folliculitis signs

and symptoms may include formation of black pustules and

spots on the skin of exposed areas. Ingestion may result in nausea, vomiting

and/or diarrhoea.

Aggravated Medical		: Pre-existing medical conditions of the following organ(s) or

Condition			  organ system(s) may be aggravated by exposure to this material: Skin.

Environmental Hazards 	: Not classified as dangerous for the environment.

- FIRST AID MEASURES

General Information 		: Not expected to be a health hazard when used under normal conditions.

Inhalation 			: No treatment necessary under normal conditions of use. If symptoms

persist, obtain medical advice.

Skin Contact 			: Remove contaminated clothing. Flush exposed area with water

and follow by washing with soap if available. If persistent irritation occurs,

obtain medical attention. When using high pressure equipment, injection of

product under the skin can occur. If high pressure injuries occur, the

casualty should be sent immediately to a hospital. Do not wait for symptoms

to develop. Obtain medical attention even in the absence of apparent

wounds.

Eye Contact 			: Flush eye with copious quantities of water. If persistent

irritation occurs, obtain medical attention.

Ingestion 			: In general no treatment is necessary unless large quantities

are swallowed, however, get medical advice.

Advice to Physician 		: Treat symptomatically. High pressure injection injuries require

prompt surgical intervention and possibly steroid therapy, to minimise tissue

damage and loss of function. Because entry wounds are small and do not

reflect the seriousness of the underlying damage, surgical exploration to

determine the extent of involvement may be necessary. Local anaesthetics

or hot soaks should be avoided because they can contribute to

swelling, vasospasm and ischaemia. Prompt surgical decompression,

debridement and evacuation of foreign material should be performed under

general anaesthetics, and wide exploration is essential.

- FIRE FIGHTING MEASURES

Clear fire area of all non-emergency personnel.

Flash point 			: > 180 °C / 356 °F (COC)

Upper / lower			: Typical 1 - 10 %(V)(based on mineral oil)

Flammability or

Explosion limits

Auto ignition temperature 	: > 320 °C / 608 °F

Specific Hazards 		: Hazardous combustion products may include: A complex

mixture of airborne solid and liquid particulates and gases

(smoke). Carbon monoxide. Unidentified organic and inorganic compounds.

Suitable Extinguishing	: Foam, water spray or fog. Dry chemical powder, carbon

Media				 dioxide, sand or earth may be used for small fires only.

Unsuitable Extinguishing	: Do not use water in a jet.

Media

Protective Equipment for	: Proper protective equipment including breathing apparatus

Firefighters			  must be worn when approaching a fire in a confined space.

- ACCIDENTAL RELEASE MEASURES

Avoid contact with spilled or released material. For guidance on selection of personal protective

equipment see Chapter 8 of this Material Safety Data Sheet. See Chapter 13 for information on

disposal. Observe all relevant local and international regulations.

Protective measures 	: Avoid contact with skin and eyes. Use appropriate containment

to avoid environmental contamination. Prevent from spreading or entering

drains, ditches or rivers by using sand, earth, or other appropriate barriers.

Clean Up Methods 		: Shovel into a suitable clearly marked container for disposal or

reclamation in accordance with local regulations.

- HANDLING AND STORAGE

General Precautions 	: Use local exhaust ventilation if there is risk of inhalation of

vapours, mists or aerosols. Properly dispose of any contaminated rags or

cleaning materials in order to prevent fires. Use the information in this data

sheet as input to a risk assessment of local circumstances to help determine

appropriate controls for safe handling, storage and disposal of this material.

Handling 			: Avoid prolonged or repeated contact with skin. Avoid inhaling

vapour and/or mists. When handling product in drums, safety footwear

should be worn and proper handling equipment should be used.

Storage 			: Keep container tightly closed and in a cool, well-ventilated

place. Use properly labelled and closeable containers. Storage

Temperature: 0 - 50 °C / 32 - 122 °F

Recommended Materials 	: For containers or container linings, use mild steel or high

density polyethylene.

Unsuitable Materials 	: PVC.

Additional Information	 : Polyethylene containers should not be exposed to high

temperatures because of possible risk of distortion.

Additional Information 	: Due to the product's semi-solid consistency, generation of

mists and dusts is unlikely to occur.

Exposure Controls 		: The level of protection and types of controls necessary will vary depending upon

potential exposure conditions. Select controls based on a risk assessment of local

circumstances.Appropriate measures include: Adequate ventilation to control

airborne concentrations. Where material is heated, sprayed or mist formed, there is

greater potential for airborne concentrations to be generated.

Personal Protective 		:Personal protective equipment (PPE) should meet recommended national standards.

Check with PPE suppliers.

Respiratory Protection 	: No respiratory protection is ordinarily required under normal conditions of use. In

accordance with good industrial hygiene practices, precautions should be taken to

avoid breathing of material. If engineering controls do not maintain airborne

concentrations to a level which is adequate to protect worker health, select

respiratory protection equipment suitable for the specific conditions of use and

meeting relevant legislation. Check with respiratory protective equipment suppliers.

Where air-filtering respirators are suitable, select an appropriate combination of

mask and filter. Select a filter suitable for combined particulate/organic gases and

vapours [boiling point >65°C(149 °F)].

Hand Protection 		: Where hand contact with the product may occur the use of gloves approved to

relevant standards (e.g. Europe: EN374, US: F739) made from the following

materials may provide suitable chemical protection: PVC, neoprene or nitrile rubber

gloves. Suitability and durability of a glove is dependent on usage, e.g. frequency

and duration of contact, chemical resistance of glove material, glove thickness,

dexterity. Always seek advice from glove suppliers. Contaminated gloves should

be replaced. Personal hygiene is a key element of effective hand care. Gloves must

only be worn on clean hands. After using gloves, hands should be washed and dried

thoroughly. Application of a non-perfumed moisturizer is recommended.

Eye Protection			: Wear safety glasses or full face shield if splashes are likely to occur.

Protective Clothing 		: Skin protection not ordinarily required beyond standard issue work clothes.

Monitoring Methods 		: Monitoring of the concentration of substances in the breathing zone of workers or in

the general workplace may be required to confirm compliance with an OEL and

adequacy of exposure controls. For some substances biological monitoring may also

be appropriate.

Environmental Exposure	: Minimise release to the environment. An environmental assessment must be made

Controls			 to ensure compliance with local environmental legislation.

- PHYSICAL AND CHEMICAL PROPERTIES

Appearance 			: Transparant Brown. Semi-solid.

Odour 				: Slight hydrocarbon.

pH 				: Not applicable.

Initial Boiling Point and	: Data not available

Boiling Range

Dropping point 		: < 200 °C

Flash point 			: > 180 °C

Upper / lower Flammability	: Typical 1 - 10 %(V) (based on mineral oil)

or Explosion limits

Auto-ignition temperature 	: > 320 °C

Vapour pressure 		: < 0.5 Pa at 20 °C

Density 			: Typical 900 kg/m3 at 15 °C

Water solubility 		: Negligible.

n-octanol/water partition	: > 6 (based on information on similar products)

coefficient (log Pow)

Kinematic viscosity		 : Not applicable.

Vapour density (air=1) 	: > 1 (estimated value(s))

Evaporation rate (nBuAc=1)	 : Data not available